Contents

1-	Design specification	6

a.	Basic value	6

b.	Design spec	6

c.	Limit	6

d.	Mechanical Specification	6

18mm=709 mil	 140mm=5512 mil 21.4mm=843 mil 40mm=1575 mil	6

37.5mm=1476 mil	59mm=2323 mil 128mm=5039 mil	6

Grow 10mm higher	6

2-	Calculation	7

a.	Power Analysis	7

i.	16V	8

ii.	5V	8

L_Encoder: 5/1K*2=10mA	8

R_Encoder: 5/1K*2=10mA	8

USB V_Bus: 5/200K=0.025mA	8

Total: 10+10+0.025=20.025mA	8

iii.	3.3V	8

Microcontroller: <250mA	8

Others: 0.2mA	8

Mosfet Driver1: 4mA	8

Mosfet Driver2: 4mA	8

Mosfet Driver3: 4mA	8

Total:250+0.2+12=262.2mA	8

iv.	Heat	8

1.	Power Diode	8

2.	Microcontroller	8

3.	Passive components circuit	8

4.	H-bridge	8

5.	8

b.	Mosfet	9

c.	H-bridge cap	9

d.	Back emf sensing	9

e.	Big-Cap Charging circuit	9

f.	H-bridge driver circuit	10

g.	Zener Diode	14

h.	Pre charge circuit calculation	14

P=U0220R	14

3-	Components choice	14

a.	Current sensing	14

i.	ZXCT1009	14

ii.	ZXCT1012	15

4-	Data variables	15

Typedef struct {int16_t data[3]; } MOTOR_DATA_3EL_16BI;	16

Typedef struct {int16_t data[2]; } MOTOR_DATA_2EL_16BI;	16

Typedef struct {int32_t data[3]; } MOTOR_DATA_3EL_32BI;	16

Typedef struct {int32_t data[2]; } MOTOR_DATA_3EL_32BI;	16

Typedef struct {int8_t data[3];} MOTOR_DATA_3EL_8BI;	16

Typedef struct{float data[4][3]; } MOTOR_DATA_CTRL;	16

#	16

Variable Name	16

Comments	16

Read/Write	16

Type	16

Range/Element Range	16

REG_MOTOR_VELOCITY	16

0—	Motor 1  velocity	16

1—	Motor 2 velocity	16

2—	Motor 3  velocity	16

DEVICE_WRITE	16

MOTOR_DATA_3EL_16BI	16

-1023~+1023	16

REG_FAN_VELOCITY	16

0—	Fan 1 velocity	16

1—	Fan 2 velocity	16

DEVICE_WRITE	16

MOTOR_DATA_2EL_16BI	16

0~1023	16

REG_MOTOR_FB_RPM	16

0—	Motor1 RPM	16

1—	Motor2 RPM	16

DEVICE_READ	16

MOTOR_DATA_2EL_32BI	16

REG_FLIPPER_FB_POSITION	16

0—	Pot 1	16

1—	Pot 2	16

DEVICE_READ	16

MOTOR_DATA_2EL_16BI	16

0~1023	16

REG_MOTOR_FB_CURRENT	16

0—	Motor 1 current	16

1—	Motor 2 current	16

2—	Motor 3 current	16

DEVICE_READ	16

MOTOR_DATA_3EL_16BI	16

0~1023	16

REG_MOTOR_ENCODER_COUNT	16

0—	Motor 1 Encoder Count	16

1—	Motor 2 Encoder Count	16

DEVICE_READ	16

MOTOR_DATA_2EL_32BI	16

REG_MOTOR_KP	16

REG_MOTOR_KP[i][j]	16

i: control mode	16

j: 0-Motor1, 1-Motor 2, 2-Motor 3	16

DEVICE_WRITE	16

MOTOR_DATA_CTRL	16

REG_MOTOR_KI	17

REG_MOTOR_KI[i][j]	17

i: control mode	17

j: 0-Motor1, 1-Motor 2, 2-	17

Motor 3	17

DEVICE_WRITE	17

MOTOR_DATA_CTRL	17

REG_MOTOR_KD	17

REG_MOTOR_KD[i][j]	17

i: control mode	17

j: 0-Motor1, 1-Motor 2, 2-	17

Motor 3	17

DEVICE_WRITE	17

MOTOR_DATA_CTRL	17

REG_MOTOR_CTRL_MODE	17

0—	Motor 1  ctrl mode	17

1—	Motor 2 ctrl mode	17

2—	Motor 3  ctrl mode	17

DEVICE_WRITE	17

MOTOR_DATA_3EL_16BI	17

REG_MOTOR_FAULT_FLAG	17

0—	Motor 1 driver fault	17

1—	Motor 2 driver fault	17

DEVICE_READ	17

MOTOR_DATA_2EL_8BI	17

REG_MOTOR_TEMP	17

0—	Thermistor 1	17

1—	Thermistor 2	17

2—	On board thermal chip	17

DEVICE_READ	17

MOTOR_DATA_3EL_16BI	17

REG_PWR_BAT_VOLTAGE	17

0—	CELL_A voltage	17

1—	CELL_B voltage	17

DEVICE_READ	17

MOTOR_DATA_2EL_16BI	17

REG_PWR_TOTAL_CURRENT	17

DEVICE_READ	17

Uint16_t	17

0~1023	17

5-	Pin Configuration	17

Available Pins:	19

Reserved Pins:	19

6-	Part list	22

- Design specification

  - Basic value

Battery voltage: 10~16.5V

Max Current: 10.9A

Highest PWM driving frequency: 10K

  - Design spec

System input voltage: 24V

Max Current: 20A

  - Limit

Battery max current limit: 18A per cell

Motor cut off current: 21A

Surge cut off time: 30ms

  - Motors:

Driving Motor: 770-8525F

http://www.nichibo-motor.com/proType_e.php?proType=65

Flipper motor: HD3SFN-6035

http://www.nichibo-motor.com/proType_e.php?proType=53

- Calculation

  - Power Analysis

    - 16V

    - 5V

## L_Encoder: 5/1K*2=10mA

## R_Encoder: 5/1K*2=10mA

## USB V_Bus: 5/200K=0.025mA

## Total: 10+10+0.025=20.025mA

    - 3.3V

## Microcontroller: <250mA

## Others: 0.2mA

## Mosfet Driver1: 4mA

## Mosfet Driver2: 4mA

## Mosfet Driver3: 4mA

## Total:250+0.2+12=262.2mA

    - Heat

      - Power Diode

      - Microcontroller

      - Passive components circuit

      - H-bridge

Current : 20A

Field discharge time:<5us

Power: (assume current change is linear)

P=WT=U∙0Ti∙dtPeriod=U∙I∙t-IT∙12∙t2|T0Period=UIT2Period=UITf2

U-voltage drop on the diode

I-max current flowing into the diode

T-field collapse time

f- driving PWM signal frequency

P=WT=UIT=≤UI dtT=0.5∙05(10-2t)dt100=0.125W

  - Mosfet

Max stall current of motors is 10.9A, so the mosfet should be able to support 20A at least. Drain-to-source voltage should be around 30V.

  - H-bridge cap

  - Back emf sensing

Voltage divider:

Scale battery voltage to 3.3 logic level:

R12R12+R11=3.316R12R11=3312.7≈14

R12=10K(5%)

R11=39K(5%)

Vbemf=16×10*1.0510*1.05+39*0.95=3.533V>3.3V

R12=10K(5%)

R11=43K(5%)

Vbemf=16×10*1.0510*1.05+43*0.95=3.272V<3.3V

  - Big-Cap Charging circuit

C=QU

I=dQdt=C∙dUdt

U=16∙(1-e-τt)

I=C∙16τ∙e-τt

Imax=16Cτ=16C∙1RC=16R<10AR>1.6 ohm

R=3.6 ohm

Choose C=10mF

Rising time: TRise≈5∙RC=5×3.6×0.01=0.18s

Fully charge timeTCharge≈10×3.6×0.01=0.36s

Try 1mF,10mF,100mF,1000mF,

R18<3.3-1Ileak=2.310×10-6=230K

  - H-bridge driver circuit

Choose RGate=100 ohm

t1=RGate∙Cgs+Cgd∙ln11-VGPVGS=16.13μs

t3=VDS-VF∙RGate∙CgdVGS-VGP=173μs

TON=t1+t3=189.13μs

t4=RGate∙Cgd+CgslnVGSVGP=0.64μs

t5=RGate∙CgdlnVDS-VFVGP=5.43μs

t6=RGate∙Cgd+CgslnVGPVTH=2.08μs

Toff=t4+t5+t6=8.15μs

CBOOT=QGate×20VBoot=140×10-9×2010=0.27μF

Choose CBOOT=0.27μF

tCHARGE=CBOOT×∆V100=CBOOT×QGATE100×CBOOT=270×140×10-9100×0.27×10-6=1.4μs

CREG≥50×CBOOT=13.5μs   (20 is not enough according to experiment)

choose CREG=22μs

  - Zener Diode

Voltage drop: smaller than N-mosfet Vds 30V, bigger than the max battery voltage plus two catch diodes’ voltage drop: 16.5+0.5*2=17.5V

  - Pre charge circuit calculation

## P=U0220R

- Components choice

  - Current sensing

Accuracy, speed, range, repeatability, cost

    - ZXCT1009

When I=20A, R_sense=0.001 ohm, V_sense=20mV, I_out_max=240uA,

R_out=3.3/I_out_max=13750 ohm,

Choose R_out=130K ohm

Testing result:

R=43K, 1.3V-2.38A

R=43K/(22/3.3/(2.38/1.3))=11.8K

    - ZXCT1012

- Data variables

# Typedef struct {int16_t data[3]; } MOTOR_DATA_3EL_16BI;

# Typedef struct {int16_t data[2]; } MOTOR_DATA_2EL_16BI;

# Typedef struct {int32_t data[3]; } MOTOR_DATA_3EL_32BI;

# Typedef struct {int32_t data[2]; } MOTOR_DATA_3EL_32BI;

# Typedef struct {int8_t data[3];} MOTOR_DATA_3EL_8BI;

# Typedef struct{float data[4][3]; } MOTOR_DATA_CTRL;

| # # | # Variable Name | # Comments | # Read/Write | # Type | # Range/Element Range |
| --- | --- | --- | --- | --- | --- |
|  | # REG_MOTOR_VELOCITY | - Motor 1 velocity - Motor 2 velocity - Motor 3 velocity | # DEVICE_WRITE | # MOTOR_DATA_3EL_16BI | # -1023~+1023 |
|  | # REG_FAN_VELOCITY | - Fan 1 velocity - Fan 2 velocity | # DEVICE_WRITE | # MOTOR_DATA_2EL_16BI | # 0~1023 |
|  | # REG_MOTOR_FB_RPM | - Motor1 RPM - Motor2 RPM | # DEVICE_READ | # MOTOR_DATA_2EL_32BI |  |
|  | # REG_FLIPPER_FB_POSITION | - Pot 1 - Pot 2 | # DEVICE_READ | # MOTOR_DATA_2EL_16BI | # 0~1023 |
|  | # REG_MOTOR_FB_CURRENT | - Motor 1 current - Motor 2 current - Motor 3 current | # DEVICE_READ | # MOTOR_DATA_3EL_16BI | # 0~1023 |
|  | # REG_MOTOR_ENCODER_COUNT | - Motor 1 Encoder Count - Motor 2 Encoder Count | # DEVICE_READ | # MOTOR_DATA_2EL_32BI |  |
|  | # REG_MOTOR_KP | # REG_MOTOR_KP[i][j] # i: control mode # j: 0-Motor1, 1-Motor 2, 2-Motor 3 | # DEVICE_WRITE | # MOTOR_DATA_CTRL |  |
|  | # REG_MOTOR_KI | # REG_MOTOR_KI[i][j] # i: control mode # j: 0-Motor1, 1-Motor 2, 2- # Motor 3 | # DEVICE_WRITE | # MOTOR_DATA_CTRL |  |
|  | # REG_MOTOR_KD | # REG_MOTOR_KD[i][j] # i: control mode # j: 0-Motor1, 1-Motor 2, 2- # Motor 3 | # DEVICE_WRITE | # MOTOR_DATA_CTRL |  |
|  | # REG_MOTOR_CTRL_MODE | - Motor 1 ctrl mode - Motor 2 ctrl mode - Motor 3 ctrl mode | # DEVICE_WRITE | # MOTOR_DATA_3EL_16BI |  |
|  | # REG_MOTOR_FAULT_FLAG | - Motor 1 driver fault - Motor 2 driver fault | # DEVICE_READ | # MOTOR_DATA_2EL_8BI |  |
|  | # REG_MOTOR_TEMP | - Thermistor 1 - Thermistor 2 - On board thermal chip | # DEVICE_READ | # MOTOR_DATA_3EL_16BI |  |
|  | # REG_PWR_BAT_VOLTAGE | - CELL_A voltage - CELL_B voltage | # DEVICE_READ | # MOTOR_DATA_2EL_16BI |  |
|  | # REG_PWR_TOTAL_CURRENT |  | # DEVICE_READ | # Uint16_t | # 0~1023 |

- Pin Configuration

|  | Pin Name | Pin # | Description | Funtion | Input/Output | Digital/Analog | Comment |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | RP20/PMRD/CN14/RD5 | 53 | L_Encoder_A | IC | Input | Digital | IC1 |
| 2 | RP25/PMWR/CN13/RD4 | 52 | L_Encoder_B | IC | Input | Digital | IC2 |
| 3 | AN14/CTPLS/RP14/PMA1/CN32/RB14 | 29 | M1_AHI | IO | Output | Digital |  |
| 4 | RP16/USBID/CN71/RF3 | 33 | M1_ALO | PWM | Output | Digital | OC1 |
| 5 | RTCC/DMLN/RP2/CN53/RD8 | 42 | M1_BHI | IO | Output | Digital |  |
| 6 | AN15/RP29/REFO/PMA0/CN12/RB15 | 30 | M1_BLO | PWM | Output | Digital | OC2 |
| 7 | VCPCON/RP24/CN50/RD1 | 49 | M1_Fault | IO | Input | Digital |  |
| 8 | TMS/CVREF/AN10/PMA13/CN28/RB10 | 23 | M1_LeftBackEMF | A/D | Input | Analog |  |
| 9 | TDO/AN11/PMA12/CN29/RB11 | 24 | M1_RightBackEMF | A/D | Input | Analog |  |
| 10 | TCK/AN12/PMA11/CTED2/CN30/RB12 | 27 | M1_Current | A/D | Input | Analog |  |
| 11 | RP22/PMBE/CN52/RD3 | 51 | R_Encoder_A | IC | Input | Digital | IC3 |
| 12 | DPH/RP23/CN51/RD2 | 50 | R_Encoder_B | IC | Input | Digital | IC4 |
| 13 | C1IND/RP21/PMA5/CN8/RG6 | 4 | M2_AHI | IO | Output | Digital |  |
| 14 | C1INC/RP26/PMA4/CN9/RG7 | 5 | M2_ALO | PWM | Output | Digital | OC3 |
| 15 | C2IND/RP19/PMA3/CN10/RG8 | 6 | M2_BHI | IO | Output | Digital |  |
| 16 | RP27/PMA2/C2INC/CN11/RG9 | 8 | M2_BLO | PWM | Output | Digital | OC4 |
| 17 | PMD5/CN63/RE5 | 1 | M2_Fault | IO | Input | Digital |  |
| 18 | PGEC3/AN5/C1INA/VBUSON/RP18/CN7/RB5 | 11 | M2_LeftBackEMF | A/D | Input |  |  |
| 19 | PGED3/AN4/C1INB/USBOEN/RP28/CN6/RB4 | 12 | M2_RightBackEMF | A/D | Input |  |  |
| 20 | AN3/C2INA/VPIO/CN5/RB3 | 13 | M2_Current | A/D | Input |  |  |
| 21 | SOSCI/C3IND/CN1/RC13 | 47 | M3_AHI | IO | Output | Digital |  |
| 22 | DMH/RP11/INT0/CN49/RD0 | 46 | M3_ALO | PWM | Output | Digital | OC5 |
| 23 | SOSCO/T1CK/C3INC/RPI37/CN0/RC14 | 48 | M3_BHI | IO | Output | Digital |  |
| 24 | RP12/PMCS1/CN56/RD11 | 45 | M3_BLO | PWM | Output | Digital | OC6 |
| 25 | AN8/RP8/CN26/RB8 | 21 | M3_Current | A/D | Input | Analog |  |
| 26 | AN9/RP9/PMA7/CN27/RB9 | 22 | M3_POSFB_1 | A/D | Input | Analog |  |
| 27 | TDI/AN13/PMA10/CTED1/CN31/RB13 | 28 | M3_POSFB_2 | A/D | Input | Analog |  |
| 28 | C3INA/CN16/RD7 | 55 | Fan1_Fail | IO | Input | Digital |  |
| 29 | C3INB/CN15/RD6 | 54 | Fan2_Fail | IO | Input | Digital |  |
| 30 | SCL3/PMD6/CN64/RE6 | 2 | FAN_I2C_SCL | I2C | Output | Digital |  |
| 31 | SDA3/PMD7/CN65/RE7 | 3 | FAN_I2C_SDA | I2C | Output | Digital |  |
| 32 | VCMPST2/CN69/RF1 | 59 | Cell_A_MOS | IO | Output | Digital |  |
| 33 | VBUSST/VCMPST1/CN68/RF0 | 58 | Cell_B_MOS | IO | Output | Digital |  |
| 34 | AN2/C2INB/VMIO/RP13/CN4/RB2 | 14 | Total_Cell_Current | AD | Input | Analog |  |
| 35 | PGEC1/AN1/VREF-/RP1/CN3/RB1 | 15 | V_Cell_A | AD | Input | Analog |  |
| 36 | PGED1/AN0/VREF+/RP0/PMA6/CN2/RB0 | 16 | V_Cell_B | AD | Input | Analog |  |
| 37 | SDA2/RP10/PMA9/CN17/RF4 | 31 | SMBUS_A_DA | I2C | Output | Digital |  |
| 38 | SCL2/RP17/PMA8/CN18/RF5 | 32 | SMBUS_A_CL | I2C | Output | Digital |  |
| 39 | DPLN/SDA1/RP4/CN54/RD9 | 43 | SMBUS_B_DA | I2C | Output | Digital |  |
| 40 | SCL1/RP3/PMCS2/CN55/RD10 | 44 | SMBUS_B_CL | I2C | Output | Digital |  |
|  |  |  | Servo2_Input | IC | Input | Digital | IC6 |
|  |  |  | Servo1_Input | IC | Input | Digital | IC5 |
|  |  |  | UART_TX | TX | Output | Digital |  |
|  |  |  | UART_RX | RX | Input | Digital |  |
|  |  |  | M3_Fault | IO | Input | Digital |  |

## Available Pins:

| 1 |  |  |  |
| --- | --- | --- | --- |
| 2 |  |  |  |

## Reserved Pins:

| 1 | MCLR | 7 |  |
| --- | --- | --- | --- |
| 2 | VSS | 9 |  |
| 3 | VDD | 10 |  |
| 4 | PGEC2/AN6/RP6/CN24/RB6 | 17 |  |
| 5 | PGED2/AN7/RP7/RCV/CN25/RB7 | 18 |  |
| 6 | AVDD | 19 |  |
| 7 | AVSS | 20 |  |
| 8 | VSS | 25 |  |
| 9 | VDD | 26 |  |
| 10 | VBUS | 34 |  |
| 11 | VUSB | 35 |  |
| 12 | D-/RG3 | 36 |  |
| 13 | D+/RG2 | 37 |  |
| 14 | VDD | 38 |  |
| 15 | OSCI/CLKI/CN23/RC12 | 39 |  |
| 16 | OSCO/CLKO/CN22/RC15 | 40 |  |
| 17 | VSS | 41 |  |
| 18 | VCAP/VDDCORE | 56 |  |
| 19 | ENVREG | 57 |  |
| 20 | PMD0/CN58/RE0 | 60 |  |
| 21 | PMD1/CN59/RE1 | 61 |  |
| 22 | PMD2/CN60/RE2 | 62 |  |
| 23 | PMD3/CN61/RE3 | 63 |  |
| 24 | PMD4/CN62/RE4 | 64 |  |

Analog Register:

| AN15 | AN14 | AN13 | AN12 | AN11 | AN10 | AN9 | AN8 | AN7 | AN6 | AN5 | AN4 | AN3 | AN2 | AN1 | AN0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1 | 0 | 0 | 0 | 0 | 0 | 0 | 1 | 1 | 0 | 0 | 0 | 0 | 0 | 0 |

I/O pin mapping:

| _TRISB15 | _TRISB14 | _TRISB13 | _TRISB12 | _TRISB11 | _TRISB10 | _TRISB9 | _TRISB8 | _TRISB7 | _TRISB6 | _TRISB5 | _TRISB4 | _TRISB3 | _TRISB2 | _TRISB1 | _TRISB0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 0 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

| _TRISC15 | _TRISC14 | _TRISC13 | _TRISC12 | _TRISC11 | _TRISC10 | _TRISC9 | _TRISC8 | _TRISC7 | _TRISC6 | _TRISC5 | _TRISC4 | _TRISC3 | _TRISC2 | _TRISC1 | _TRISC0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 0 |  |  |  |  |  |  |  |  |  |  |  |  |  |

| _TRISD15 | _TRISD14 | _TRISD13 | _TRISD12 | _TRISD11 | _TRISD10 | _TRISD9 | _TRISD8 | _TRISD7 | _TRISD6 | _TRISD5 | _TRISD4 | _TRISD3 | _TRISD2 | _TRISD1 | _TRISD0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | 0 | 0 | 0 | 0 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 0 |

| _TRISE15 | _TRISE14 | _TRISE13 | _TRISE12 | _TRISE11 | _TRISE10 | _TRISE9 | _TRISE8 | _TRISE7 | _TRISE6 | _TRISE5 | _TRISE4 | _TRISE3 | _TRISE2 | _TRISE1 | _TRISE0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  | 0 | 0 | 1 |  |  |  |  |  |

| _TRISF15 | _TRISF14 | _TRISF13 | _TRISF12 | _TRISF11 | _TRISF10 | _TRISF9 | _TRISF8 | _TRISF7 | _TRISF6 | _TRISF5 | _TRISF4 | _TRISF3 | _TRISF2 | _TRISF1 | _TRISF0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  |  |  | 0 | 0 | 0 |  | 0 | 0 |

| _TRISG15 | _TRISG14 | _TRISG13 | _TRISG12 | _TRISG11 | _TRISG10 | _TRISG9 | _TRISG8 | _TRISG7 | _TRISG6 | _TRISG5 | _TRISG4 | _TRISG3 | _TRISG2 | _TRISG1 | _TRISG0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | 0 | 0 | 0 | 0 |  |  |  |  |  |  |

- Part list